### Latching Relay:

- Power Consumption: Latching relays only consume power when switching states (ON to OFF or vice versa). Once the relay has switched states, it stays in that state without drawing any further power.

- ON/OFF Control: It can be controlled with a brief pulse to set it to ON or OFF, making it energy-efficient.

- Types: They are available as single-coil and dual-coil latching relays.

  - Single-coil: Requires a pulse to set and reset the relay.

  - Dual-coil: Has separate coils for ON and OFF states, each requiring a pulse to toggle the state.

### Benefits:

- Low Power Consumption: Once the relay is switched, no continuous power is required to maintain its state.

- Energy-Efficient: Suitable for battery-powered or energy-conscious applications like smart meters or home automation systems.

- State Retention: Keeps the last state even during power outages or disconnections.

### Use Case:

In a smart electricity metering system, where you want to manage load control or power cutoff with minimal energy use, a latching relay would be ideal. It allows control of ON/OFF states without constant power draw, enhancing the efficiency of the system.

### Example of Common Low Power Latching Relays:

- Panasonic DS2Y-S-DC5V (Low power consumption latching relay)

- Omron G6SK-2F-L (Ultra-miniature, low-power latching relay)

### Difference Between SPDT and Latch Relay:

| Features | SPDT Relay | Latching Relay |
| --- | --- | --- |
| Relay Type. | Single Pole Double Throw (SPDT) | Latching Relay (SPST) |
| Contact Configuration. | 1 NO (Normally Open), 1 NC (Normally Closed) | 1 NO (Normally Open) |
| Coil Voltage. | 5V DC. | 12V DC. |
| Current Rating. | 16-30A Max. (with appropriate safety factor) | 16-30A Max. (continuous operation) |
| Switching Mechanism. | Changes state with pulse | Maintains state until changed |
| Power Consumption (Coil). | 0.5W - 2W (depends on model) | 0.5W - 1W (only during switching) |
| Size | Compact | Depends on module |
| Operational Life | Higher | Lower as compared to SPDT |
| Price | Generally Lower | Generally Higher |

### Conclusion:

- SPDT Relays are ideal for applications that require frequent switching and immediate response. They tend to be more cost-effective and have a longer mechanical life, making them suitable for general relay applications in energy meters.

- Latching Relays are better suited for situations where maintaining the last state is critical and where power efficiency is a concern, especially in battery-operated devices. Although they are more expensive, their ability to hold the state without continuous power makes them advantageous in specific applications.