### Power Requirements:

#### 1. PZEM-004T (Metering Module)

- Description: Measures voltage, current, power, and energy consumption.

- Operating Voltage: 80-260V AC

- Current Measurement: Up to 100A

- Power Consumption: Approximately 0.5W

- Key Features: High precision, real-time monitoring.

#### 2. ESP32 WROOM 32 (Microcontroller Unit)

- Description: A versatile MCU with Wi-Fi and Bluetooth capabilities.

- Operating Voltage: 3.3V

- Peak Current: Up to 160mA during Wi-Fi operation

- Power Requirement:

P =  V × I  = 3.3 V × 0.16 A = 0.528W

- Key Features: Dual-core processor, integrated peripherals.

#### 3. DS3231 (RTC Module)

- Description: Real-time clock with temperature compensation.

- Operating Voltage: 3.3V to 5V

- Current Requirement:

  - Battery mode: 0.1µA

  - Active mode: 1-2mA

- Power Requirement (worst case):

P  = 5 V × 0.002 A = 0.01 W

#### 4. AT24C256 (EEPROM Module)

- Description: 256K I2C EEPROM for data storage.

- Operating Voltage: 1.8V to 5.5V

- Current Requirement: Approximately 1mA

- Power Requirement:

P = 5 V × 0.001 A = 0.005 W

#### 5. 20x4 LCD (Liquid Crystal Display)

- Description: Displays information in a 20x4 character format.

- Operating Voltage: 5V

- Current Requirement: Approximately 2-20mA

- Power Requirement:

P = 5 V × 0.02 A = 0.1 W

#### 6. SIM800L (GPRS Module)

- Description: GSM/GPRS module for communication.

- Operating Voltage: 3.4V to 4.4V (recommended 4.0V)

- Peak Current: Up to 2A during transmission

- Power Requirement:

P = 4 V × 2 A = 8 W

#### 7. ZMPT101B (Voltage Sensor Module)

- Description: Measures AC voltage.

- Operating Voltage: 5V

- Current Requirement: Typically 10-20mA

- Power Requirement:

P = 5 V × 0.02 A = 0.1 W

#### 8. A1220 Hall Effect Sensor Module

- Description: Detects magnetic fields.

- Operating Voltage: 4.5V to 24V

- Current Requirement: Typically 10-15mA

- Power Requirement:

P = 5 V × 0.015 A = 0.075 W

#### 9. D2440-B (Solid State Relay)

- Description: Controls AC loads, ideal for high power applications.

- Operating Voltage: 5V DC control, 24-280V AC load

- Control Current: Typically 10-15mA

- Power Requirement:

P = 5 V × 0.015 A = 0.075 W

### 10. TCA9548A I2C Multiplexer

- Description: An 8-channel I2C switch that allows multiple I2C devices to communicate on the same bus, managing communication between up to eight I2C devices with the same address.

- Operating Voltage: 1.65V to 5.5V

- Current Requirement: Typically 10mA

- Power Requirement:

P = 5 V × 0.01 A = 0.05 W

Power Requirement Table:

| Module | Voltage(V) | Current(Amp) | Power(W) |
| --- | --- | --- | --- |
| PZEM-004T | 5 | 0.1 | 0.5 |
| ESP32 WROOM 32 | 3.3 | 0.16 | 0.53 |
| DS3231 | 5 | 0.002 | 0.01 |
| AT24C256 | 5 | 0.001 | 0.005 |
| 20x4 LCD | 5 | 0.02 | 0.1 |
| SIM800L | 4 | 2 | 8 |
| ZMPT101B | 5 | 0.02 | 0.1 |
| A1220 | 5 | 0.015 | 0.075 |
| D2440-B | 5 | 0.015 | 0.075 |
| TCA9548A | 5 | 0.01 | 0.05 |
| Total | - | - | 9.445 |

### On the basis of above power calculations of System we decided to use HLK-PM05 Hi-Link 5V 20W AC to DC Power Supply Module:

The HLK-PM05 is a compact and efficient AC to DC power supply module that converts 85-265V AC input to a regulated 5V DC output. This module is designed to provide reliable power for various electronic applications, making it ideal for powering low-power devices such as microcontrollers, sensors, and communication modules. With a high power density and small form factor, it is suitable for integration into smart energy metering systems and other compact electronic devices.

### Specifications

| Specification | Details |
| --- | --- |
| Input Voltage | 85-265V AC |
| Output Voltage | 5V DC |
| Output Current | 4A (max) |
| Output Power | 20W (max) |
| Efficiency | > 82% |
| Ripple Voltage | < 50mV |
| Operating Temperature | -40°C to +85°C |
| Size | 60mm x 36mm x 18mm |
| Weight | 60g |
| Protection Features | Short Circuit, Overload, Over Temperature Protection |