### 1. PZEM-004T Metering Module

- Voltage Range: 80–260V, Resolution: 0.1V, Accuracy: 0.5%.

- Current Range: 0–10A or 0–100A, Starting Current: 0.01A (10A), 0.02A (100A).

- Power Factor Range: 0.00–1.00, Accuracy: 1%.

- Power Range: 0–2.3kW (10A) or 0–23kW (100A), Resolution: 0.1W.

- Operating Temperature: -20°C to +60°C​

### 2. DS3231 RTC Module

- Accuracy: ±2 minutes/year (0°C–40°C), ±3.5 minutes/year (-40°C–85°C).

- Battery Backup: CR2032, keeps time during power outages.

- Interface: I2C, Speed: Up to 400kHz.

- Operating Voltage: 3.3V–5.5V​

### 3. ESP32 WROOM 32 MCU

- Processor: Dual-core Xtensa LX6, Clock Speed: 240 MHz.

- Memory: 520KB SRAM, up to 8MB Flash.

- Connectivity: Wi-Fi (802.11 b/g/n) and Bluetooth (BLE).

- GPIO Pins: 34 GPIO, ADC: 12-bit SAR ADC with up to 18 channels.

- Operating Temperature: -40°C to 125°C​

### 4. AT24C256 EEPROM

- Memory Size: 256Kb (32KB), Interface: I2C.

- Clock Frequency: Up to 1 MHz, Operating Voltage: 1.7V–5.5V.

- Data Retention: 100 years​

### 5. SIM800L GPRS Module

- Network: Quad-band GSM/GPRS (850/900/1800/1900 MHz).

- Data Transfer Rate: GPRS Class 12.

- Operating Voltage: 3.4V–4.4V.

- UART Interface: Up to 115.2 kbps.

### 6. ZMPT101B Voltage Sensor

- Voltage Measurement Range: Up to 240V AC.

- Output: Analog, proportional to measured AC voltage.

- Accuracy Class: 0.2, Operating Temp: -40°C to 85°C​

### 7. A3144 Hall Effect Sensor

- Operating Voltage: 3.3V–24V.

- Response Time: 2µs, Supply Current: 3.5mA at 5V.

- Magnetic Sensitivity: ±65 Gauss, Output Type: Digital (open-drain)​

### 8. 20x4 LCD Display

- Resolution: 20x4 characters.

- Interface: I2C, reducing pin usage.

- Operating Voltage: 5V, with adjustable backlight.

### SK90-S-DC5V-1C Relay Module

- Coil Voltage: 5V DC.

- Contact Rating: 10A at 250V AC / 30V DC.

- Switching Power: 2500VA (AC), 300W (DC).

- Switching Time: Operate: 10ms, Release: 5ms.

- Operating Temperature: -40°C to +85°C.

### Hi-Link HLK-5M05 5V SMPS

- Input Voltage: 90–264V AC or 100–370V DC.

- Output Voltage: 5V DC, Current: 4A (20W).

- Efficiency: Up to 80%, Ripple & Noise: ≤50mV.

- Protection: Short-circuit, over-current, over-voltage.

- Isolation Voltage: 3000V AC, Certifications: CE, UL, RoHS.

### TCA9548A I2C Multiplexer

- Voltage Range: 1.65V to 5.5V.

- I2C Buses: 8 channels.

- I2C Speed: Up to 1MHz (Fast-mode plus).

- Addressing: Configurable (0x70 to 0x77).

- Control: Channel selection via I2C.

- Operating Temp: -40°C to +85°C.