SMPS Module Comparison:

| Specification | Easy Power SMPS 5V 1A | Hi-Link HLK-5M05 5V 1A |
| --- | --- | --- |
| Brand | Easy Power. | Hi-Link |
| Model. | 5V 1A AC to DC SMPS Power Supply Module | HLK-5M05 Mini PCB Mounted SMPS Power Supply Module |
| Input Voltage Range. | 110V – 260V AC. | 90V – 265V AC. |
| Output Voltage. | 5V DC. | 5V DC. |
| Output Current. | 800mA continuous, up to 1A for short durations (10 min max) | 1A continuous. |
| Output Power. | 4W (5V × 0.8A). | 5W (5V × 1A). |
| Efficiency. | ~75%. | 75-80%. |
| Ripple & Noise. | 200 - 600 mV (20 MHz bandwidth) | ≤50mV (20 MHz bandwidth) |
| Output Voltage Accuracy. | ±0.2% | ±0.1%. |
| Load Regulation. | ±0.5%. | ±0.5%. |
| Line Regulation. | ±0.5%. | ±0.2%. |
| Operating Temperature Range. | 0°C to +50°C. | -25°C to +60°C. |
| Protection Features. | Short-circuit, Over-voltage. | Short-circuit, Over-current, Over-voltage, Self-recovery |
| Isolation Voltage. | 1000V AC. | 3000V AC. |
| Dimensions. | 37.00mm × 24.00mm × 21.00mm | 34mm × 22mm × 18mm |
| Weight. | 22g. | 32g. |
| Cost | Rs. 150-250 | Rs. 250-400 |

### Conclusion:

- Hi-Link HLK-5M05 is a more reliable and robust option, especially for applications requiring higher accuracy, better protection, and compliance with safety standards. It also offers better performance in terms of ripple, noise, and operating temperature range. While it is slightly more expensive, its features make it a good value for the price.